Alneelain University

Graduate College of Studies

The SPEED CONTROL OF DC MOTOR CONTROLLER USING LINEAR QUADRATIC REGULATOR (LQR)SYSTEM

A Thesis submitted in partial fulfillment of the requirement for the Degree of M.Sc.in Electrical Control Engineering

Proposed by

EZAR  RAMADAN  HASAAN

Supervisor

Dr: ABDUALRAHIM.ELATEE

NOVAMBER 2018

الآية

بسم الله الرحمن الرحيم

"يرفع الله الذين امنوا منكم والذين أوتوا العلم درجات والله بما تعملون خبير"

صدق الله العظيم

(سورة المجادلة :11)

Dedication

This is dedicated to family ,teachers and with special regarded to research supervisor  Dr. Abd ALRAHIM .ELATEE and also would like to extend sensed of gratitude and thanks to whoever extend a helping hand in research.

ACKNOWLEDGMENT

Wished to thank committee members who more than generous with their expertise and precious time . A special thank to Dr.ABDALRAHIM .ELATEE research supervisor for his countless hours of reflecting ,reading, encouraging and most of all patience throughout the entire reach period . would like to acknowledge and thank members of faculty of postgraduate for allowing to conduct my research and providing any assistance requested . Special thank went to the member of Faculty of Engineering for their continued support . Their excitement and willingness to provided  feedback made the completion of this research an enjoyable experience.

ABSTRA

This research the  Speed Control of DC Motor Controller using Linear Quadratic Requlator (LQR) System ,the objective is desing Linear  Quadratic Regulator in Matlab evnviroment to given better prefomance step responace.The resultes is given small step response and better preformance.

المستخلص

لقد قدمت متحكمه باستخدام تقنيه المقوم الخطي زو الدرجه التانيه  لتحكم في سرعه محرك تيار مستمر والهدف هو تصميم متحكمة المقوم الخطى زو الدرجه الثانيه باستخدام برنامج ماتلاب لتحقيق اداء افضل بدراسة الاستجابه الزمنيه .حيث اثبتت النتائج ان زمن الاستجابه اقل بالتالي تحقق اداء افضل.

Contents

الايه	I

Dedication	I

ACKNOWLEDGMENT	iII

ABSTRAC	v

المستخلص	Iv

Contents	vI

List of figures	XII

List of tables	X

Chapter one	1

INDTRODACTION	1

1.1 Preface.....................	1

1.3 Problem Statement	1

1.4 Objectives.......................	1

1.5 Contribution of thesis:	2

Chapter two	3

Literature review	3

1.2 Background.................	3

2.2 DC MOTOR......................	4

2.2.1 DC MOTOR PRINCIPLE 	4

2.2.2 BACK OR COUNTER E.M.F.	5

2..3 SIGNIFICANCE OF BACK E.M.F	5

2.2.4  DC MOTOR TYEP	5

SHUNT WOUND MOTOR  	6

SERIES WOUND MOTOR	7

COMPOUND WOUND MOTOR	7

2.2.5  PART OF DC MOTOR	10

2.2.6  COMMUTATION IN DC MOTOR	10

2.2.7 VOLTAGE EQUATION OF MOTORS	10

2.2.8 CONDITION FOR MAXIMUM POWER	11

2.2.9 ARMATURE TORQUE OF DC MOTOR	12

2.2.10 SPEED OF DC MOTOR	13

2.2.11SPEED RELATIONS	14

2.2.12 APPLICATION OF DC MOTOR 	15

ARmTURE REACTION	15

2.2.13 APPLCATION OF DC MOTOR	15

2.3 optimal control	16

2.4 PLANT..........................	16

2.5 Performance Index	17

2.6 Constraints................	20

2.7 Formal Statement of Optimal Control System	20

2.8 Linear Quadratic Regulator	22

2.8.1 INTRODUTION	22

2.8.2 OPTIMAL REGULATOR DESIGN	25

2.8.3  LINEAR QUADRTIC TRACKING SYSTEM	26

2.8.4  DESINING LQR  CONTROLLER	28

2.8.5  APPLICATION OF LINEAR QUADRATIC REGULATOR	31

Chapter three	32

Mathematical Modelling	32

3.1  MODLING OF DC MOTOR	32

3.1 .1 ELECTRICAL CHARACTERSTIC	33

3.1.2 MECHANICAL CHARACTERSTIC	33

3.1 STATE- SPACE REPRESENTION  	33

3.5Linear Quadratic Regulator (LQR) Applied in System	34

Chapter four	37

Implementations	37

4.1 introduction...........	37

4.2 Results AND DISCUSSION	37

Chapter five	42

Conclusion and recommendations	42

5.2 Recommendations................	42

References	43

Appendix	44

List of Figures

figuer (2.1) Fleming Left Hand Rule..............................................................4

Figure (2.2) Back or Counter EMF.....................................................................5

Figure (2.3)shunt wount motor...........................................................................6

Figure (2.4) serier wount motor...............................................................................7

Figure (2.5) short-shunt connection.........................................................................8

Figure (2.6)long-shunt connection...........................................................................9

Figure (2.7) voltage equation of motor..................................................................10

Figure (2.8)armature torque of motor...........................................................12

Figure (2.9) modern control configuration...............................................16

Figure(2.10)optimal control problem.........................................................21

Figure (2.11) implementation of the optimal tracking syst...................28

figure (2.12) locus of closed-loop pole locations for the signal degree of freedom oscillator ......................................................................29

Figure (3.1) ELECTRICAL CIRCUIT REPRESNTATION OF A DC MOTOR................................................................................................................32

Figure (4.1) STEP RESPONSE OF DC MOTOR OPEN LOOP....................................37

Figure (4.2) STEP RESPONSE OF DC MOTOR WITH LQR.......................................38

Figure (4.3) STEP RESPONSE WHEN R = 1.1...................................................39

Figure (4.4) STEP RESPONSE WHEN R = 0.5...................................................40

Figure (4.5) STEP RESPONSE WHEN R = 0.01.................................................41

List Of  Tables

Table (3.1) parameters of DC motor.................................................................33

Table (4.1) CHANGE OF THE R,K,AND RESPONSE........................................41

CHAPTER One

Introduction

1.1. Preface

Now a day’s DC (Directed Current) motors are widely used in industrial applications, the speed of DC motors can be controlled by various driver circuits. In which the drives application mainly involves complex process such as modeling, control, simulation and parameters tuning .

Optimal control theory is a branch of mathematics developed to find optimal ways to control a dynamic system as a smart variable speed drive. It is deals with the problem of optimization of smart variable speed drive . The problem must be well posed before any solution can be attempted. This requires a clear The aim is to arrive at a mathematical description of the smart variable speed drive which is simple enough to deal with, and realistic enough to be able to predict the response of the system to any given input.[1]

1.2.PROBLEM  FORMALATION

The speed of the DC motor may change due to disturbance present surrounding it. This will make the desired speed sometimes change and will be not maintain. By using LQR control algorithm, the deviation of the speed can be optimized.

1.3.Project Objectives

The objectives of this project are as follows:

- To implement Linear Quadratic Regulator (LQR) controller using  matlab.

(ii) To develop the dc motor controller using Linear Quadratic Regulator (LQR) algorithm.

1.4.Contribution of thesis

The major contribution of this thesis are summarized as follows:

- Chapter one: present introduction about project, problem statements, objective and contribution of thesis.

- Chapter two: background, present general concept of the DC motor, the optimal control and linear quadratic regulator.

- Chapter three: present the mathematical model of the system.

- Chapter four: present the result and discussion of model.

- Chapter five: present conclusion and recommendations of the thesis.

Chapter Tow

Literature review

2.1.BACKGRONT

Aamir Hashim Obeid Ahmed. Studied  “Optimal Speed Control for Direct Current Motors Using Linear Quadratic Regulator” . The compare of time response specification between conventional Proportional-Integral-Derivatives (PID) controller and Linear Quadratic Regulator (LQR) for a speed control of a separately excited DC motor, to determine which control strategy delivers better performance with respect to DC motor’s speed. Performance of these controllers has been verified through simulation using MATLAB/SIMULINK software package. According to the simulation results, liner quadratic regulator method gives the better performance, such as settling time, steady state error and overshoot compared to conventional PID controller. This shows the superiority of liner quadratic regulator method over conventional PID controller.[2]

Neenu Thomas, Dr. P. Poongodi . Studied “Position Control of DC Motor Using Genetic Algorithm Based PID Controller”. To design a position controller of a DC motor by selection of a PID parameters using genetic algorithm. The model of a DC motor is considered as a third order system. T The compare two kinds of tuning methods of parameter for PID controller. One is the controller design by the genetic algorithm, second is the controller design by the Ziegler and Nichols method. It was found that the proposed PID parameters adjustment by the genetic algorithm is better than the Ziegler & Nichols’ method. [3]

Mr. Tushar Upalanchiwar , Prof.A.V.Sakhare. Studied “Design and implementation of the fuzzy PID controller using MATLAB/SIMULINK model ”. They Proposed PID controller with fuzzy technology. i.e. fuzzy PID controller, here them are analyze the performance of the conventional PID controller using the MATLAB/SIMULINK model. It were found the performance of fuzzy PID controller is better than conventional PID controller in controlling speed, and time response of specifications.[4]

Tarmizi Bin Ahmed Izzuddin,Zamani Bin Md ,FauzalNaim Bin Zohedi Sani . Studied “Controller Design for Vector Controlled AC Induction Motor Drive using State-space Design Methodologies” . Proposed the application of modern state-space controller design towards a linearised AC induction motor drive using state feedback linearization methods. By doing so many simple and effective controller (based on modern state-space methodologies) such as the quadratic optimal controller, feedback with feed forward controller etc, could be apply towards an AC induction motor drive.[5]

2.2.DC motor

Although AC motors are used in most of the cases, DC motors have many applications and used for multi-purpose applications.

2.2.1.DC Motor Principle

A machine that converts DC power into mechanical energy is known as DC motor. Its operation is based on the principle that when a current carrying conductor is placed in a magnetic field, the conductor experiences a mechanical force. The  direction of the force is given by Fleming’s left hand rule. [6]

Fig(2.1)Fleming Left Hand Rule

2.2.2.Back or Counter EMF

When the armature of a DC motor rotates under the influence of the driving torque, the armature conductors move through the magnetic field and hence an E.M.F (Back Electromotive Force). is induced in them. The induced E.M.F. acts in opposite direction to the applied voltage V(Lenz’s law) and is known as back or counter E.M.F. Eb.

Fig(2.2) Back or Counter EMF

2.2.3.Significance of Back E.M.F

The presence of back E.M.F. makes the DC motor a self-regulating machine i.e., it makes the motor to draw as much armature current as is just sufficient to develop the torque required by the load. Back E.M.F. in a DC motor regulates the flow of armature current i.e., it automatically changes the armature current to meet the load requirement.[6]

2.2.4.DC Motor Types

• Shunt Wound

• Series Wound

• Compound wound

Shunt Wound Motor

In shunt wound motor the field winding is connected in parallel with armature. The current through the shunt field winding is not the same as the armature current. Shunt field windings are designed to produce the necessary M.M.F. by means of a relatively large number of turns of wire having high resistance. Therefore, shunt field current is relatively small compared with the armature current.[6]

Fig(2.3) Shunt Wound Motor

The equation of the circuit:-

Itotal= total current in armature = load current

Ia=armature current

Ish =shund field current

E=applied voltage

Eb=back ectromotiv force

Ra=armture resitance

Itotal=Ia+Ish

Where

Or,

Ia=Itotal+Ish=ERa

General voltage equation of DC shunt motor is

E=Eb+IaRa

Or,

E=Eb+(Itotal_Ish)Ra

Series Wound Motor

In series wound motor the field winding is connected in series with the armature. Therefore, series field winding carries the armature current. Since the current passing through a series field winding is the same as the armature current, series field windings must be designed with much fewer turns than shunt field windings for the same M.M.F. Therefore, a series field winding has a relatively small number of turns of thick wire and, therefore, will possess a low resistance. [7]

Fig(2.4) Series Wound Motor

Itotal=Ise=Ia

E=Eb+IseRse+IaRse

Or,

E=Eb+Ia(Ra+Rse)

3-Compound Wound Motor

Compound wound motor has two field windings; one connected in parallel with the armature and the other in series with it. There are two types of compound motor connections

1) Short-shunt connection

2) Long shunt connection

When the shunt field winding is directly connected across the armature terminals it is called short-shunt connection.[7]

Fig(2.5)Short-Shunt connection

Ise=series current

Rse =sries rsistance

Itotal=Ia+Ish

Series motor

Ia=Ise

Therefore, the current equation of DC motor is given by

Itotal=Ise+Ish…

And its voltages equation is ,

E=Eb+Ia(Ra+Rse)

When the shunt winding is so connected that it shunts the series combination of armature and series field it is called

long-shunt connection.

Fig(2.6)Long-shunt connection

Shunt field current is given as

Ish=ERsh

Series field current is given as

Ise=Ia=IL+Ish

Terminal voltage is given as

V=Ee-IaRa-IseRse=Eg-Ia(Ra+Rse)

If the brush contact drop is included ,terminal voltage equation is written as

V=Eg-IaRa+Rse-2Vb

Power devolped=EgIa

Power outpu= VIL

2.2.5.Parts of a DC Motor

A simple motor has six parts

• Armature/ Rotor.

• Commutator.

• Brushes.

• Axle.

• Permanent Magnet.

• DC Power supply.

2.2.6.Commutation in DC Motor

In order to produce unidirectional force (or torque) on the armature conductors of a motor, the conductors under any pole must carry the current in the same direction at all times. The function of commutator and brush gear in a DC motor is to cause the reversal of current in a conductor as it moves from one side of a brush to the other.[7]

2.2.7.Voltage Equation of Motors

Let in a DC motor

V = applied voltage

Eb = back E.M.F.

Ra = armature resistance

Ia = armature current

Since back E.M.F. Eb acts in opposition

to the applied voltage V, the net

voltage across the armature circuit is VE

b. The armature current Ia is given by

Fig(2.7) Voltage Equation of Motors

V=applid voltage

Ia=V-EbRa                                                                       (2.1)

V=Eb+IaRa                                                                       (2.2)

This is known as voltage equation of the D.C. motor. [7]

2.2.8.Condition For Maximum Power

The mechanical power developed by the motor is   Pm=EbIaSince, V and Ra are fixed, power developed by the motor depends upon armature current. For maximum power, dPm/dIa should be zero.

∴  dpmdIa=V-2IaRa=0                                       (2.3)

Or

IaRa=V2                                                                       (2.4)

Now,

V=Ea+Ia  Ra=Eb+V2                                      (2.5)

∴    Ea=V2                                                                           (2.6)

Hence mechanical power developed by the motor is maximum when back E.M.F. is equal to half the applied voltage.[7]

2.2.9.Armature Torque of a DC Motor

Torque is the turning moment of a force about an axis and

is measured by the product of force (F) and radius at right angle to which the force acts.

T=armuter torue

T=F*r

Fig(2.8) Armature Torque of a DC Motor

In DC motor, each conductor is acted upon by circumferential force, F at a distance r.

i=IaA , B=∅a                                                                        2.7

a=2πϠ/p

=Z×∅2πrl/P×IaA×l×r=Z∅IaP2πAN-m                            (2.8)

Ta=0.159Z∅IaPAN-m                                                           (2.9)

Since Z,P,andA are fixed for agiven machine,[7]

∴ Ta∝∅Ia

2.2.10.Speed of a DC Motor

Ea=V-IaRa

But

Eb=P∅ZN  60 A

∴       P∅ZN60 A=V-IaRa

Or

N=(V-IaRa)∅60 APZ                              (2.10)

Or

N=K(V-IaRa)∅                                    (2.11)

Where

k=60 APZ

But

V=IaRa=Ea

∴                          N=KEa∅                            (2.12)

Or

N∝Eb∅                                                                 (2.13)

Therefore, in a DC motor speed is directly proportional to back E.M.F., Eb and inversely proportional to flux.[7]

2.2.11.Speed Relations

N1∝Eb1∅1              and                N2∝Eb2∅2

∴     N2N1=Eb2Eb1×∅1∅2                                              (2.14)

- For a shut motor ,flux practically remains constant so that ∅1=∅2.

∴       N2N1=Eb2Eb1                                                           (2.15)

- For a series motor, ∅∝Ia  prior to saturation.

∴    N2N1=Eb2Eb1×Ia1 Ia2                                             (2.16)

Where                      Ia1=    initial armature current

Ia2=   final armature current

Armature Reaction

In DC machine, the main field is produced by the field coils. In both the generating and motoring modes, the armature carries current and magnetic field is established, which is called armature flux. The effect of armature flux on main field is called the armature reaction. [8]

armature reaction does

It demagnetizes or weakens the magnetic flux/field. It cross-magnetizes or distorts it.[8]

Overcome

The demagnetizing effect can be overcome by adding extra ampere-turns on the main field.

The cross magnetizing effect can be reduced by having common poles.[8]

2.2.12. Application of DC motor

1-Shunt Motor

The characteristics of a shunt motor reveal that it is an approximately constant speed motor. Industrial Use: Lathes, Drills, Boring Mills, Shapers , Spinning, and weaving machines.[9]

2-Series Motor

It is a variable speed motor i.e. speed is low at high torque and vice versa. This motor is used when large starting torque is required. Industrial Use: Electric Traction Cranes, Elevators, hair drier, Sewing machine.[9]

2.3.Optimal Control

The main objective of optimal control is to determine control signals that will cause a process (plant) to satisfy some physical constraints and at the same time extremize (maximize or minimize) a chosen performance criterion (performance index or cost function). Referring to Figure (2.9),we are interested in finding the optimal control u*(t) (* indicates optimal condition) that will drive the plant P from initial state to final state with some constraints on controls and states and at the same time extremizing the given performance index J we are interested in finding the optimal control u*(t) (* indicates optimal condition) that will drive the plant P from initial state to final state with some constraints on controls and states and at the same.[10]

The formulation of optimal control problem requires

1. a model of the process to be controlled.

2. a specification of the performance index.

3. astatement of boundary conditions and the physical constraints on the controls.

Figure (2.9):Modern Control Configuration

2.4 	Plant

For the purpose of optimization, we describe a physical plant by a set of linear or nonlinear differential or difference equations. For example, a linear time-invariant system is described by the state and output relations (2.17) and (2.18) and a nonlinear system by (2.19) and (2.20).

2.5	Performance Index

Classical control design techniques have been successfully applied to linear, time-invariant, single-input, single output (SISO) systems. Typical performance criteria are system time response to step or ramp input characterized by rise time, settling time, peak overshoot, and steady state accuracy; and the frequency response of the system characterized by gain and phase margins, and bandwidth. In modern control theory, the optimal control problem is to find a control which causes the dynamical system to reach a target or follow a state variable (or trajectory) and at the same time extremize a performance index which may take several forms as described below[10].

- Performance Index for Time-Optimal Control System: We try to transfer a system from an arbitrary initial state x(to) to a specified final state x(t f) in minimum time. The corresponding performance index (PI) is

J= t0tfdt=tf-t0=t*                     (2.17)

- Performance Index for Fuel-Optimal Control System: Consider a spacecraft problem. Let u(t) be the thrust of a rocket engine and assume that the magnitude u(t)of the thrust is proportional to the rate of fuel consumption. In order to minimize the total expenditure of fuel, we may formulate the performance index as:

J=t0tfu(t)dt                                     (2.18)

For several controls, we may write it as

J=t0tfi=0mRiui(t)dt    (2.19)

where R is a weighting factor

- Performance Index for Minimum-Energy Control System: Considerui(t)as the current in the ith loop of an electric network. Then i=1mui2(t)ri(where, ris the resistance of the ith loop) is the total power or the total rate of energy expenditure of the network. Then, for minimization of the total expended energy, we have a performance criterion as

J=t0tfi=1mui2(t)ridt                                             2.20

Or in general

J=t0tfu'tRutdt                                          2.21

where, R is a positive definite matrix and prime (') denotes transpose here Similarly, we can think of minimization of the integral of the squared error of a tracking system. We then have

J=t0tfx'tQxtdt                                              (2.22)

where, Xd(t)is the desired value, xa(t) is the actual value, and x(t) = xa(t) - Xd(t), is the error. Here, Q is a weighting matrix, which can be positive semi-definite.

- Performance Index for Terminal Control System: In a terminal target problem, we are interested in minimizing the error between the desired target position Xd (tf) and the actual target position Xa (tf) at the end of the maneuver or at the final time t f. The terminal (final) error is x ( tf) = Xa ( tf) - Xd ( tf). Taking care of positive and negative values of error and weighting factors, we structure the cost function as

J=X'tfFxtf                                                          (2.23)

which is also called the terminal cost function. Here, F is a positive semi-definite matrix.

- Performance Index for General Optimal Control System: Combining the above formulations, we have a performance index in general form as

J=X'tfFtf+t0tfx;tQxt+u'tRu(t)       (2.24)

J=Sxtf,tf+t0tfVxt,ut,t dt                         (2.25)

Where, R is a positive definite matrix, and Q and F are positive semidefinite matrices, respectively. Note that the matrices Q and R may be time varying. The particular form of performance index (2.24) is called quadratic (in terms of the states and controls) form. The problems arising in optimal control are classified based on the structure of the performance index J . If the PI (2.25) contains the terminal cost function S(x(t), u(t), t) only, it is called the Mayer problem, if the PI (2.25) has only the integral cost term, it is called the Lagrange problem, and the problem is of the Bolza type if the PI contains both the terminal cost term and the integral cost term as in (2.25). There are many other forms of cost functions depending on our performance specifications[5]. However, the above mentioned performance indices (with quadratic forms) lead to some very elegant results in optimal control systems

2.6	Constraints

The control u( t)and state x( t)vectors are either unconstrained or constrained depending upon the physical situation. The unconstrained problem is less involved and gives rise to some elegant results. From the physical considerations, often we have the controls and states, such as currents and voltages in an electrical circuit, speed of a motor, thrust of a rocket, constrained as

U+≤ut≤U-andX-≤xt≤X+      (2.26)

where, +, and - indicate the maximum and minimum values the variables can attain

2.7	Formal Statement of Optimal Control System

Let us now state formally the optimal control problem even risking repetition of some of the previous equations. The optimal control problem is to find the optimal control u*(t) (* indicates extremal or optimal value) which causes the linear time-invariant plant (system)

X=Axt+Bxt                        (2.27)

to give the trajectory x* (t) that optimizes or extremizes (minimizes or maximizes) a performance index

J=X'tfFtf+t0tfx'tQxt+u'tRu(t)dt     (2.28)

to give the state x*(t) that optimizes the general performance index

J=Sxtf,tf+t0tfVxt,ut,t dt                         (2.29)

with some constraints on the control variables u( t) and/or the state variables x(t) given by (2.29). The final time t f may be fixed, or free,

Figure (2.10):Optimal Control Problem

and the final (target) state may be fully or partially fixed or free. The entire problem statement is also shown pictorially in Figure (2.10)Thus,

we are basically interested in finding the control u*(t) which when applied to the plant described by (2.27) or (2,29), gives an optimal performance index J* described by (2.28) or (2.30). The optimal control systems are studied in three stages.

- In the first stage, we just consider the performance index of the form (2.30) and use the well-known theory of calculus of variations to obtain optimal functions.

- In the second stage, we bring in the plant (2.27) and try to address the problem of finding optimal control u*(t) which will  drive the plant and at the same time optimize the performance index (2.28). Next, the above topics are presented in discrete time domain. 3. Finally, the topic of constraints on the controls and states (2.26) is considered along with the plant and performance index to obtain optimal control.

2.8. linear quadratic regulator

2.8.1 .Introduction

Linear quadratic regulator (LQR) problem is a special type of optimal control that deals with linear systems (in state and in control) and minimization of objective or cost function that are quadratic or the quadratic performance index.[11]

Singh and Gupta considered LQR control in (2011)for a scalar system when the sensor, controller and actuator all have their own clocks that may drift apart from each other. They also considered both an affine and a quadratic clock model. They analyzed the loss of performance incurred as a function of how asynchronous the clocks are for a quadratic cost function. In the end, they were able to obtain guidelines on how often to utilize the communication resources to synchronize the clocks[11].

Review of past work showed that Alberto B. et al. presented a technique to compute the explicit state-feedback solution to both the finite and infinite horizon linear quadratic optimal control problem subject to state and input constraints. They succeeded in showing that the control law is piece-wise linear and continuous for both the finite horizon problem (model predictive control) and the usual infinite time measure (constrained linear quadratic regulation). As a practical consequence of the result, constrained linear quadratic regulation becomes attractive also for systems with high sampling rates, as on-line quadratic programming solvers are no more required for the implementation.

Similarly, linear quadratic regulators (LQR) were applied by Polyaka and Tempob in 2001 to study robust design of uncertain systems in a probabilistic setting. Systems affected by random bounded nonlinear uncertainty were considered so that classical optimization methods based on linear matrix inequalities cannot be used without conservatism. The approach used was a blend of randomization techniques for the uncertainty along with convex optimization for the controller parameters.

In Costa’s work, he suggested that a linear feedback law can be used to improve the dynamic response of the converter in small signal models of quasi-resonant (QR) converters with current-mode control, obtained by state-space averaging. He added that this can be done by seeking a gain vector that places the closed-loop poles at the locations desired. Instead, he used optimal control techniques, together with those small-signal models, to design a robust voltage controller for QR converters that overcomes some uncertainties that may corrupt the results obtained by classical design methods .

In Rantzer and Johansson’s work titled: “Piecewise Linear Quadratic Optimal Control”, the use of piecewise quadratic cost functions was extended from stability analysis of piecewise linear systems to performance analysis and optimal control. Lower bounds on the optimal control cost were obtained by semi-definite programming based on the Bellman inequality. This also gave an approximation to the optimal control law. A compact matrix notation was also introduced to support the calculations and it was proven that the framework of piecewise linear systems could be used to analyze smooth nonlinear dynamics with arbitrary accuracy.[11]

In 2010, Swigart and Lall developed controller synthesis algorithms for decentralized control problems, where individual subsystems were connected over a network. Their work was focused on the simplest information structure, consisting of two interconnected linear systems, and they constructed the optimal controller subject to a decentralization constraint via a spectral factorization approach. They provided explicit state-space formulae for the optimal controller, characterized its order, and show that its states are those of a particular optimal estimator [7]. By applying the over bounding method of Petersen and Hollot in 1994, Douglas and Athans derived a linear quadratic regulator that is robust to real parametric uncertainty. The resulting controller was determined from the solution of a single modified Riccati equation. This controller has the same guaranteed robustness properties as standard linear quadratic designs for known systems. It is proven that when applied to a structural system, the controller achieves its robustness by minimizing the potential energy of uncertain stiffness elements, and minimizing the rate of dissipation of the uncertain damping elements.

Previously, Safonov and Athans had explained in their work in (1976) that multi-loop linear-quadratic state-feedback (LQSF) regulators demonstrated robustness against a variety of large dynamical, time-varying, and nonlinear variations in open-loop dynamics. The results were interpreted in terms of the classical concepts of gain and phase margin, thus strengthening the link between classical and modern feedback theory.

More recently (2013), Lamperski and Cowan, in their paper titled: “Time-changed Linear Quadratic regulators”, gave a solution to the linear quadratic regulator problem in which the state is perfectly known, but the controller’s measure of time is a stochastic process derived from a strictly increasing L´evy process. They concluded that the optimal controller is linear and can be computed from a generalization of the classical Riccati differential equation. [11]

2.8.2 Optimal Regulator Design

The object of the optimal regulator design is to determine the optimal control law u*(x,t)which can transfer the system from its initial state to the final state (with zero system input) such that a given performance index is minimized. The performance index is selected to give the best trade-off between performance and cost of control. The performance index that is widely used in optimal control design is known as the quadratic performance index and is based on minimum-error and minimum-energy criteria. Consider the plant described by[12]

x't=Axt+But                       2.30

The problem is to find the vector K(t) of the control law

ut=-ktxt                        (2.31)

which minimizes the value of a quadratic performance index J of the form

J=t0tfx'Qx+u'Ru dt      (2.32)

subject to the dynamic plant equation in (2.30). In (2.32) Q is a positive semidefinite matrix and R is a real symmetric matrix. Q is positive semidefinite if all its principal minors are nonnegative. The choice of the elements of Q and R allows the relative weighting of individual state variables and individual control inputs.

To obtain a formal solution, we can use the method of Lagrange multipliers. The constraint problem is solved by augmenting (2.30) into (2.32) using an n-vector of Lagrange multipliersλ. The problem reduces to the minimization of the following unconstrained function[7].

Lx,λ,u,t=x'Qx+u'Ru+λ'[Ax+Bu-x']                 (2.33)

The optimal values (denoted by the subscript *) are found by equating the partial derivatives to zero.

∂L∂λ=Ax'+Bu'-x'=0⟹ x'=Ax'+Bu'                           (2.34)

∂L∂u=2Ru'+λ'B=0   ⟹ u'=-12R-1λ'B                            (2.35)

∂L∂x=2x'Q+λ'+λ'A=0   ⟹ λ=-2Qx'-A'λ                    (2.36)

Assume that there exists a symmetric, time varying positive definite matrix p(t) satisfying

λ=2P(t)x'                                (2.37)

Substituting (2.37) into (2.35) gives the optimal closed-loop control law

u*t=-R-1B-p(t)x'                                   (2.38)

Obtaining the derivative of (2.37), we have

λ'=2(p'x*+px'*)                                            (2.39)

p't=-ptA-A'pt-Q+ptBR-1B'p(t)              (2.40)

The above equation is referred to as the matrix Riccati equation. The boundary condition for (2.40) is p(tf ) = 0. Therefore, (2.40) must be integrated backward in time. Since a numerical solution is performed forward in time, a dummy time variable τ=tf-tt is replaced for time t. Once the solution to (2.40) is obtained the solution of the state equation (2.33) in conjunction with the optimum control equation (2.38) is obtained[12].

2.8.3 Linear Quadratic Tracking System

Given the linear, observable system see figure (2.11)

x't=Atxt+B(t)u(t)                                  (2.41)

yt=Ctx(t)                                            (2.42)

the desired output z(t), the error et=zt-y(t) and the performance index.

J=12e'tfFtfetf+12t0tf[e'tF(tf)e(tf)+u(t)'R(t)u(t)]dt                                                                                         (2.43)

Then the optimal control u*t is given by:

u*t=-Ktx*t+R-1tB'tgt                  (2.44)

Where the n×n symetcric,positive define matrix Pt is solution  of the nonlinear ,matrix deferential riccati equation (DRE).

Pt=-PtAt-A'tPt+PtEtPt-V(t)    (2.45)

With the final solution

Ptf=C'tfFtfCtf                                           (2.46)

the optimal cost J*

J*t0=12x*;t0Pt0x'(t0)-x'(t0)g(t0)+h(t0)z         (2.47)

The implementation of the tracking system is shown in Figure (2.11)

Figure (2.11) Implementation of the Optimal Tracking System

2.8.4 Designing LQR Controller

While the above section presents all the formulas needed to start designing MIMO LQR controllers, the point of this section is to inform the potential user of the limitations of the methodology. The most restrictive aspects of LQR controllers is that they are full-state feedback controllers. This means that every state that appears in the model of the physical system Equation must be measured by a sensor. In fact, the notation of z(t) = C x(t) for the linear combination of states that are to be regulated to zero is deliberate. We do not call z(t) the outputs of the system because all the states x(t) must be measured in real time to implement the control law .Full-state feedback is appropriate and can be applied to systems whose  Dynamics are described by a finite set of differential equations and whose states can readily be measured. An aircraft in steady, level flight is an example of a system whose entire state can be measured with sensors. In fact, LQR control has been used to design flight control systems for modern aircraft.

On the other hand, full-state feedback is typically not appropriate for flexible systems. The dynamics of flexible systems are described by partial differential equations that often require very high-order, if not infinite-dimensional, state-space models. As such, it is not feasible to measure all the states of systems that possess flexible dynamics. Returning to the aircraft example, one could not use a LQR controller to regulate the aerodynamically induced vibrations of the aircraft’s wing. The number of sensors that wouldbe needed to measure the state of the vibrating wing prohibit this.

Figure (2.12):Locus of closed-loop pole locations for the single degree of freedom oscillator

Having discussed the implications of full-state feedback, the next restrictive aspect of LQR to discuss is the gap between what the LQR controller achieves and the desired control system performance. Recall that the LQR is the control that minimizes the quadratic cost subject to the constraints imposed by the system dynamics. This optimization problem and the resulting optimal controller have very little to do with more meaningful control system specifications like levels of disturbance rejection ,overshoot in tracking, and stability margins. This gap must always be kept in mind when using LQR to design feedback controllers. The fact that LQR controllers are in some sense optimal is of no consequence if they do not meet the performance goals. Further since control system specifications are not given in terms of minimizing quadratic costs, it becomes the job of the designer to use the LQR tool wisely. To do so it helps to adopt a means-to-an-end design philosophy where the LQR is viewed as a tool, or the means, used to achieve the desired control system performance, or the end.

One last issue to point out is that LQR controller design is an iterative process even though the methodology systematically produces optimal, stabilizing controllers. Since the LQR formulation does not directly allow one to achieve standard control system specifications, trial and error iteration over the values of the weights in the cost is necessary to arrive at satisfactory controllers.

Typically LQR designs are carried out by choosing values for the design weights, synthesizing the control law, evaluating how well the control law achieves the desired robustness and performance, and iterating through this process until a satisfactory controller is found. In the sequel, various properties and weight selection tools will be presented that provide good physical and mathematical guidance for selecting values for the LQR design variables. Yet these are only guides, and as such they will not eliminate the iterative nature of LQR controller design[13].

Application of optimal control

The major thrust is on applications which include: aerospace marine and automotive systems structural and mechanical design robots and manufacturing systems chemical and industrial processes electric power systems and energy management operations research and business socio-economic models biomedical systems environmental control and ecology management and electrical and electronic systems.[1]

Chapter Three

METHODOLOGY

3.1  Modeling of DC Motor

The equivalent electrical circuit of a DC motor is illustrated in Figure (3.1). It can be represented by a voltage source (Va) across the coil of the armature. The electrical equivalent of the armature coil can be described by an inductance (L) in series with a resistance (R) in series with an induced voltage (Vc) which opposes the voltage source. The induced voltage is generated by the rotation of the electrical coil through the fixed flux lines of the permanent magnets. This voltage is often referred to as the back emf (electromotive force).

Figure (3.1) Electrical Circuit Representation Of A Dc Motor.

The physical parameters have been set up as followed

Table(3.1)parameters of DC motor . [2]

| Parameters | Values |
| --- | --- |
| Electric Resistance, R | 1Ω |
| Electric Inductance, L | 0.5 H |
| Electromotive Force Constant, K | 0.023 Nm A-1 |
| Moment of Inertia of the Rotor, J | 0.01 Kg m2 |
| Damping Ratio of the Inertia of the Rotor, B | 0.00003 Nms rad-1 |

3.1.1 Electrical characteristic

From the Figure 3.1, the following equations based on Newton’s Law and Kirchoff’s Law has been had.

diadt=RLia-KLωr+1LVa                                                   (3.1)

dωrdt=KJia-BJωr                                                                (3.2)

3.1.2 Mechanical characteristic

The motor torque, T is related to the armature current, ia by a constant factor, K and can be written as;

T=Kia                                                               (3.3)

3.1.3 State-space Representation

In the state-space form, the equations above can be expressed by choosing the rotating speed and electrical current as the states variables and the voltage as an input. The output is chosen to be the rotating speed

diadtdωrdt=-RL-KLKJ-BJiaωr+1L0va      =Ax+Bu          3.4

y=01iaωr+0va =Cx+Du                (3.5)

By substituting the parameters on the state-space model, the value of A, B, C and D can be getting.

diadtdωrdt=-2.0000-0.04602.3000-0.0030iaωr+20va

yt=01iaωr+0va

A=-2-0.04602.3-0.0030       B=20

C=01              D=0

3.2  Linear Quadratic Regulator (LQR) Applied in System

To meet the desired specification, the controller is designed using LQR methodology. Let the cost function be defined as

J=0∞(xtQx+utRu)dt                                           (3.6)

Where Q is weighting factors of states (positive semidefinite matrix) and R is weighting factors of control variables (positive definite matrix). To design the LQR controller, the first step is to select the weighting matrices Q and R. The value R weight inputs more than the states while the value of Q weight the state more than the inputs. Then the feedback K can be computed and the closed loop system responses can be found by simulation.

The LQR controller is given by

u=-kx                                             (3.7)

where K is the constant feedback gain obtained from the solution of the discrete algebraic Riccati equation. The gain matrix K which solve the LQR problem is

K=R-1BTP*                                    (3.8)

Where P* is unique, positive semidefinite solution to the Riccati equation;

AT P+PA-PBR-1BTP+Q=0                    3.9

Let:

Q=1001

R=1

Tuning of Q and R matrix, P matrix is determined. So the optimal feedback gain is obtained as:

k=R-1BTP                                 (3.22)

In LQR the cost function which is to be minimized is given by:

J=0∞XTQX+UTRUdt                                 (3.23)

The two matrices Q and R are selected by design engineer by using Bryston rule. Selecting Q large means that, to keep J small. On the other hand choosing R large means that the control input u must be smaller to keep J small One should select Q to be positive semi definite and R to be positive definite. This means that the scalar quantity XTQX is always positive or zero at each time t. The best value of the Q & R matrix is calculated by checking the step response of the system (with LQR).

Chapter Four

Implementation

4.1 introduction

Structure of Q and R parameters are needed in the determination of optimal control gain of the systems, as they vary minimization of the quadratic performance index. These structures are explicated extensively in this work. Finally, solution to linear quadratic regulator problems is achieved through the use of the MATLAB.

4.2 Results  and Discussion

Shows the step response for DC motor open loop in figure (4-1), shows the step response for DC motor with Linear Quadratic Regulator in figure (4-2).

Figure(4.1)step response of DC motor open loop

Figure(4.2)step response of DC motor with LQR

The controller is modeled in MATLAB environment, the simulation results show that the proposed controller gives better performance and less settling time .

When shows the change of R matrix the gain is change, if the gain increase to a high enough extent some system can become un stable ,but it will decrease the steady state error .Whenever the R  is small   and the  time   response is reduced. Figure (4.3), (4.4), and (4.5) explain the time  response after change the R matrix

Show the step response when R=1.1,the time response is increase .

Figure(4.3)step response when R=1.1

Figure(4.4)step response when R = 0.5

Show the step response when R=0.5the time response is decrease .

Figure(4.5)step response when R =0.01

Show the step response when R=.01,the time response is very decrease ,

Table(4.1)changes of the R,  and time response

| R | Time response |
| --- | --- |
| 1.1 | 3.80 |
| 1.0 | 3.57 |
| 0.5 | 2.60 |
| 0.01 | 2.74 |

Change the value  R is small the time response is small and performance index is better.

Chapter Five

CONCLUSION AND RCOMMENDATION

5.1. conclusion

Speed control of DC motor important issue, so this research presented design method for the selection of Q and R matrixes to determine the optimal speed control using LQR method .

The best value of the Q and R matrixes is calculated by checking the step response of the system with LQR .The result had been given best performance index .

Change the value R  is small the time response is small and performance index is better .The result  had achieved through use of matlab software .

5.2.Recommendation

After the best result of speed controller of DC motor of Linear Quadratic   Regulator using matlab.  In this researcher the disturbance is not found in the system .In future studies  it is recommendation to use system with disturbance and compare the response of the with LQR controller.

Reference

[1] www.rseachgate.net.

[2]Aamir Hashim Obeid Ahmed“Optimal Speed Control for Direct Current Motors Using Linear Quadratic Regulator” Journal of Science and Technology - Engineering and Computer Sciences, Vol. 14, No. 2, December/ 2013.

[3] Neenu Thomas, Dr. P. Poongodi.“Position Control of DC Motor Using Genetic Algorithm Based PID Controller" Proceedings of the World Congress on Engineering 2009 Vol II WCE 2009,July 1-3,2009,London,U.K.

[4] Mr. Tushar Upalanchiwar , Prof.A.V.Sakhare. “Design and implementation of the fuzzy PID controller using MATLAB/SIMULINK  model ”. International Journal of Research in Computer and Communication Technology, Vol 3, Issue 3, March- 2014.

[5] Tarmizi Bin Ahmad Izzuddin1, a,Zamani Bin Md Sani2,b

FauzalNaim Bin Zohedi3,c“Controller Design for Vector Controlled AC Induction Motor Drive using State-space Design Methodologies” Advanced Materials Research Vol. 903 (2014) .

[6]http:// dept . ru . Ac. Bd .

[7] circitglobe.com .

[8]www.electricaleasy.com.

[9]www.courshro.com.

[10]	D. S. Naidu, Optimal control systems: CRC press, 2002.

[11]	A. A. Ajasa and A.-A. A. Sebiotimo, "THE USE OF MATLAB IN THE SOLUTION OF LINEAR QUADRATIC REGULATOR (LQR) PROBLEMS."

[12]	H. Saadat and E. E. Series, Computational aids in control systems using MATLAB: McGraw-Hill, 1993.

[13]	W. S. Levine, "The Control Systems Handbook: Control System Advanced Methods," 2010 .

Appendix

Clear all;

Clc;

display('------------Linear Quadratic Regulator-----------------')

t = 0:0.001:10;

J = 0.01;

b = 0.00003;

K = 0.023;

R = 1;

L = 0.5;

A = [-R/L -K/L;K/J -b/J];

B = [1/L;0];

C = [0 1];

D = [0];

step(A,B,C,D)

Q = diag([1 1]);

R = 1;

[K,s,e] = lqr(A,B,Q,R);

AC = A- B*K;

step(AC,B,C,D,1,t)

R = 1.1;

[K,s,e] = lqr(A,B,Q,R);

AC = A- B*K;

step(AC,B,C,D,1,t)

R = 0.5;

[K,s,e] = lqr(A,B,Q,R);

AC = A- B*K;

step(AC,B,C,D,1,t)

R = 0.01;

[K,s,e] = lqr(A,B,Q,R);

AC = A- B*K;

step(AC,B,C,D,1,t)